# Histogram Equalization and Contrast Stretching

## Objective

- To enhance the visual quality of an image using histogram equalization and contrast stretching techniques.

- To compare the effectiveness of built-in vs custom histogram equalization.

- To observe the effect of different contrast stretching transformations and compare them with histogram equalization.

## 1️⃣ Histogram Equalization

### a) Concept

Histogram equalization improves image contrast by redistributing pixel intensity values.

- Dark or washed-out images often have pixel intensities clustered in a small range.

- HE spreads these intensities across the full range (0–255), enhancing image details.

### b) Built-in Histogram Equalization

- Used a library function to automatically equalize the image histogram.

- Observed a contrast-enhanced image where details in dark and bright regions became more visible.

### c) Custom Histogram Equalization

Performed manually following class steps:

- Compute the histogram of the image (frequency of each pixel intensity).

- Calculate the Cumulative Distribution Function (CDF) from the histogram.

- Normalize the CDF to map intensity values to 0–255.

- Transform the original image using the normalized CDF.

Observation:

- The manually equalized image closely matched the built-in function result.

- This step-by-step process helped understand how HE works internally.

## 2️⃣ Contrast Stretching

### a) Concept

Contrast stretching expands the range of pixel intensities using linear or non-linear transformations, improving image visibility.

### b) Applied Transformations

- Linear Stretching: Maps minimum intensity to 0 and maximum to 255.

  - Enhances overall contrast in a straightforward way.

- Logarithmic Transformation: Expands darker pixels more than brighter ones.

  - Highlights details in dark regions.

- Gamma (Power-law) Transformation: Adjusts image brightness and contrast using a gamma value.

  - Helps correct lighting conditions.

Observation:

- Each transformation emphasized different intensity ranges.

- Some transformations made dark areas brighter, while others preserved overall balance.

## 3️⃣ Comparison of Results

| Enhancement Technique | Observation |
| --- | --- |
| Built-in Histogram Equalization | Quick and automatic; enhances contrast uniformly. |
| Custom Histogram Equalization | Manual method; results similar to built-in; shows internal working. |
| Linear Contrast Stretching | Simple; expands full intensity range; may not enhance local details. |
| Log Transformation | Brightens dark regions; enhances low-intensity details. |
| Gamma Transformation | Flexible control over brightness and contrast; can correct lighting issues. |

Conclusion:

- Histogram Equalization is ideal for uniform contrast enhancement.

- Contrast Stretching provides more control over specific intensity ranges.

- Comparing the methods helps choose the most suitable technique based on image characteristics.

## 4️⃣ Skills Learned

- Understanding the histogram and its importance in image processing.

- Applying built-in and manual histogram equalization.

- Implementing contrast stretching using linear, logarithmic, and gamma transformations.

- Comparing different techniques for image enhancement.

- Observing the impact of enhancement on image details and visual quality.